GOSHEN TOWNSHIP

Clermont County, Ohio

PROPERTY MAINTENANCE

CODE

July 2007

Department of Community & Economic Development

6757 Goshen Road Goshen, Ohio 45122

The Property Maintenance Code
of Goshen Township

CHAPTER 1

ADMINISTRATION

SECTION 101

GENERAL

101.1 	Title. These regulations shall be known as the Property Maintenance Code of Goshen Township, 	hereinafter referred to as “this code”.

101.2 	Scope. The provisions of this code shall apply to all existing residential and nonresidential structures and all existing premises and constitute minimum requirements and standards for premises and structures for sanitation and protection from the elements, life safety, safety from fire and other hazards, and for safe and sanitary maintenance; the responsibility of owners, operators and occupants, the occupancy of existing structures and premises, and for the administration, enforcement and penalties.

101.3	Intent.  This code shall be construed to secure its expressed intent, which is to ensure public health, safety and welfare is so far as they are affected by the continued occupancy and maintenance of structures and premises.  Existing structures and premises that do not comply with these provisions shall be altered or repaired to provide a minimum level of health and safety as required herein.

101.4	Severability. If a section, subsection, sentence, clause or phrase of this code is, for any reason, held to be unconstitutional, such decision shall not affect the validity of the remaining portions of this code.

SECTION 102

APPLICABILITY

102.1 	General.  The provisions of this code shall apply to all matters affecting or relating to structures and premises, as set forth in Section 101.  Where, in a specific case, different sections of this code specify different requirements, the most restrictive shall govern.

102.2 	Maintenance.  Equipment, systems, devices, and safeguards required by this code or a previous regulation or code under which the structure or premises was constructed, altered, or repaired shall be maintained in good working order. No owner, operator, or occupant shall cause any service, facility, equipment, or utility which is required under this section to be removed or shut off from, or discontinued for any occupied dwelling, except for such temporary interruption as necessary while repairs or alterations are in progress. The requirements of this code are not intended to provide the basis for removal or abrogation of fire protection and safety systems and devices in existing structures. Except as otherwise specified herein, the owner or the owner’s designated agent shall be responsible for the maintenance of buildings, structures and premises.

102.3	Application of other codes.  Repairs, additions or alterations to a structure, or changes or changes of occupancy, shall be done in accordance with the provisions of the Clermont County Building Code and the Goshen Township Zoning Resolution as applicable.

102.4	Existing remedies.  The provisions of this code shall not be construed to abolish or impair existing remedies of Goshen Township or its officials or agencies relating to the removal or demolition or any structure which is dangerous, unsafe and unsanitary.

102.5 	Workmanship. Repairs, maintenance work, alterations or installations which are caused directly or indirectly by the enforcement of this code shall be executed and installed in a workmanlike manner and installed in accordance with the manufacturer’s installation instructions.

102.6 	Historic buildings. The provisions of this code shall not be mandatory for existing buildings or structures designated as historic buildings when such buildings or structures are judged by the code official to be safe and in the public interest of health, safety and welfare.

102.7	Reserved.

102.8 	Requirements not covered by code. Requirements necessary for the strength, stability or proper operation of an existing fixture, structure or equipment, or for the public safety, health and general welfare, not specifically covered by this code, shall be determined by the code official.

SECTION 103

DEPARTMENT OF COMMUNITY & ECONOMIC DEVELOPMENT

  - General.   The Goshen Township Department of Community & Economic Development shall be responsible for the enforcement and application of this code and the executive official in charge shall be known as the code official.

  - Appointment.	The code official shall be appointed by the Goshen Township Board of Trustees; and the code official shall not be removed from office except for cause and after full opportunity to be heard on specific and relevant charges by and before the appointing authority.

  - Deputies.  In accordance with the prescribed procedures of Goshen Township with the concurrence of the Goshen Township Board of Trustees, the code official shall have the authority to appoint a deputy code official, other related technical officers, inspectors and other employees.

  - Liability.  The code official, officer or employee charged with the enforcement of this code, whole acting for Goshen Township shall not thereby be rendered liable personally, and is hereby relieved from all personal liability for any damage accruing to persons or property as a result of an act required or permitted in the discharge of official duties.

Any suit instituted against any officer or employee because of an act performed by that officer or employee in the lawful discharge or duties and under the provisions of this code shall be defended by the legal representation of Goshen Township until the termination of the proceedings.  The code official or any subordinate shall not be liable for costs in an action, suit or proceeding that is instituted in pursuance of the provisions of this code:; any employee of the Department of Community & Economic Development, acting in good faith and without malice, shall be free from liability for acts performed under any of its provisions or by reasons of any act or omission in the performance of official duties in connection therewith.

SECTION 104

DUTIES AND POWERS OF THE CODE OFFICIAL

  - General.   The code official shall enforce the provisions of this code

104.2 	Rule-making authority. The code official shall have the authority, with the advice and consent of the Goshen Township Board of Trustees, as necessary in the interest of the public health, safety and general welfare, to adopt and promulgate rules and procedures; to interpret and implement the provisions of the code; to secure the intent thereof; and to designate requirements applicable because of local climatic or other conditions.  Such rules shall not have the affect of waiving structural requirements specifically provided for in this code, or of violating accepted engineering methods involving public safety.

104.3 	Inspections. The code officer shall make all of the required inspections, or shall accept reports of inspection by approved agencies or individuals.  All reports of such inspections shall be in writing and be certified by a responsible officer of such approved agency or by the responsible individual. The code officer is authorized to engage such expert opinion as deemed necessary to report upon unusual technical issues that arise, subject to the approval of the appointing authority.

104.4 	Right-of-entry. The code official is authorized to enter a structure or premises at reasonable times to inspect subject to constitutional restrictions on unreasonable searches and seizures.  If entry is refused or not obtained, the code official is authorized to pursue recourse as provided by law.

104.5	Identification. The code official shall carry proper identification when inspecting structures or premises in the performance of duties under this code.

104.6   Notices and orders.  The code official shall issue all necessary notices or orders to ensure compliance with this code.

104.7	Department records. The code official shall keep official records of all business and activities of the department specified in the provisions of this code. Such records shall be retained in the official records in such manner and for so long as is required by the Goshen Township retention policy.

SECTION 105

APPROVAL

105:1	Modifications.  Whenever there are practical difficulties involved in carrying out the provisions for individual cases, provided the code official shall first find that special individual reason makes the strict letter of this code impractical and the modification is in compliance with the intent and purpose of this code and that such modification does not lessen health, life and fire code requirements.  The details of action granting modifications shall be recorded and entered in the department files.

  - Alternative materials, methods and equipment.  The provisions of this code are not intended to prevent the installation of any material or to prohibit any method of construction not specifically prescribed in this code, provided that any such alternative has been approved.  An alternative material or method of construction shall be approved where the code official finds that the proposed design is satisfactory and complies with the intent of the provisions of this code, and that the material, method or work offered is, for the purpose intended, at least the equivalent of that prescribed in this code in quality, strength, effectiveness, fire resistance, durability and safety.

  - Required Testing.  Whenever there is insufficient evidence of compliance with the provisions of this code, or evidence that a material or method does not conform to the requirements of this code, or in order to substantiate claims for alternative materials or methods, the code official shall have the authority to require tests to be made as evidence of compliance at no expense to Goshen Township.

    - Tests methods.  Test methods shall be as specified in this code or by other recognized test standards.  In the absence of recognized and accepted test methods, the code official shall be permitted to approve appropriate testing procedures performed by an approved agency.

    - Test reports.  Reports of tests shall be retained by the code official for the period required for retention of public records.

  - Material and equipment reuse.  Materials, equipment and devices shall not be reused unless such elements are in good repair or have been reconditioned and tested when necessary, placed in good  and proper working condition and approved.

SECTION 106

VIOLATIONS

106.1 	Unlawful Acts. It shall be unlawful for a person, firm or corporation to be in conflict with or in violation of any of the provisions of this code.

  - Notice of violation.  The code official shall serve a notice of violation or order in accordance with Section 107.

  - Prosecution of violation.  Any person failing to comply with a notice of violation or order served in accordance with Section 107 shall be deemed guilty of a minor misdemeanor and the violation shall be deemed a strict liability offense.  If the notice of violation is not complied with, the code official shall institute the appropriate proceedings at law or in equity to restrain, correct, or abate such violation, or to require the removal or termination of the unlawful occupancy of the structure in violation of the provisions of this code or of the order or direction made pursuant thereto.

  - Violation penalties.  Any person who shall violate a provision of this code, or fail to comply therewith, or with any of the requirements thereof, shall be prosecuted within the limits provided by state or local laws.  Each day that a violation continues after due notice has been served shall be deemed a separate offense.

  - Abatement of violation.  The imposition of the penalties herein prescribed shall not preclude Goshen Township Board of Trustees through its legal officer from instituting appropriate action to restrain, correct or abate a violation, or prevent illegal occupancy of a building, structure or premises, or to stop an illegal act, conduct, business or utilization of the building, structure or premises.

SECTION 107

NOTICES AND ORDERS

  - Notice to person responsible.  Whenever the code official determines that there has been a violation of this code or has grounds to believe a violation has occurred, notice shall be given in the manner prescribed in Section 107.2 and 107.3 to the person responsible for the violation as specified in this code.  Notices of condemnation procedures shall also comply with Section 108.3.

  - Form. Such notice prescribed in Section 107.1 shall be in accordance with all of the following:

- Be in writing;

- Include a description of the real estate sufficient for identification.

- Include a statement of the violation or violations and why the notice is being issued.

- Include a correction order allowing a reasonable time to make the repairs and improvements required to bring the dwelling unit or structure into compliance with the provisions of the code.

- Inform the property owner of the right to appeal.

- Include a statement of the right to file a lien in accordance with Section 106.3.

  - Method of service.  Such notice shall be deemed to be properly served if a copy thereof is:

- Delivered personally;

- Sent by certified or first-class mail addressed to the last known address; or

- If the notice is returned showing that the letter was not delivered, a copy thereof shall be posted in a conspicuous place in or about the structure affected by such notice.

  - Reserved.

  - Transfer of ownership.	  It shall be unlawful for the owner of any dwelling unit or structure who has received a compliance order or upon whom a notice of violation has been served to sell, transfer, mortgage, lease or otherwise dispose of such a dwelling unit or structure to another until the provisions of the compliance order or notice or violation have been complied with, or until such owner shall first furnish the grantee, transferee, mortgagee or lessee  a true copy of any compliance order or notice of violation issued by the code official and shall furnish to the code official a signed and notarized statement from the grantee , transferee, mortgagee or lessee, acknowledging the receipt of such compliance order or notice of violation and fully accepting the responsibility without condition for making the corrections or repairs required by such compliance order or notice of violation.

SECTION 108

UNSAFE STRUCTURES

  - General.  When a structure is found by the code official to be unsafe, or when a structure is found to be unfit for human occupancy, or is found unlawful, such structure shall be condemned pursuant to the provisions of this code.

    - Unsafe structure.  An unsafe structure is one that is found to be dangerous to the life, health, property or safety of the public or the occupants of the structure by not providing minimum safeguards to protect or warn occupants in the event of fire, or because such structure contains unsafe equipment or is so damaged, decayed, dilapidated, structurally unsafe or such faulty construction or unstable foundation, that partial or complete collapse is possible.

    - Reserved.

    - Structure unfit for human occupancy.  A structure is unfit for human occupancy whenever the code official finds such a structure unsafe, unlawful or, because or the degree to which the structure is in disrepair or lacks maintenance, is unsanitary, vermin or rat infested, contains filth and contamination, or lacks ventilation, illumination, sanitary or heating facilities or other essential equipment required by this code, or because the location of the structure constitutes a hazard to the occupants of the structure or to the public.

    - Unlawful structure.  An unlawful structure is one found in whole or in part t be occupied by more persons than permitted under this code, or was erected, altered or occupied contrary to law.

  - Closing of vacant structures.  If the structure is vacant and unfit for human habitation and occupancy, and is not danger of structural collapse, the code official is authorized to post a placard of condemnation on the premises and order the structure closed up so as not to be an attractive nuisance.  Upon failure of the owner to close up the premises within the time specified in the order, the code official shall cause the premises to be closed and secured through any available public agency or by contract or agreement by private persons and the cost thereof shall be charged against the real estate upon which the structure is located and shall be a lien upon such real estate and may be collected by any other legal resource.

  - Notice.  Whenever the code official has condemned a structure under the provisions of this section, notice shall be posted in a conspicuous place in or about the structure affected by such notice and served on the owner or the person or persons responsible for the structure in accordance with Section 107.3.  The notice shall be in the form prescribed in Section 107.2

  - Placarding.  Upon failure of the owner or persons responsible to comply with the notice provisions within the time given, the code official shall post on the premises a placard bearing the word “Condemned” and a statement of the penalties provided for occupying the premises or removing the placard.

    - Placard removal. The code official shall remove the condemnation placard whenever the defect or defects upon which the condemnation and placarding action were based have been eliminated.  Any person who defaces or removes a condemnation placard without approval of the code official shall be subject to the penalties provided by this code.

  - Prohibited occupancy.  Any occupied structure condemned and placarded by the code official shall be vacated as ordered by the code official.  Any person who shall occupy a placarded premises, and any owner or any person responsible for the premises who shall let anyone occupy  a placarded premises shall be liable for the penalties provided by this code.

SECTION 109

EMERGENCY MEASURES

  - Imminent danger.  When, in the opinion of the code official, there is imminent danger of failure or collapse of a building or structure which endangers life, or when any structure or part of structure has fallen and life is endangered by the occupation of the structure, or when there is actual or potential danger to the building occupants or those in proximity of any structure because of explosives, explosive fumes or vapors or the presence of toxic fumes, gases or materials, the code official is hereby authorized and empowered to order and require the occupants to vacate the premises forthwith.  The code official shall cause to be posted at each entrance to such structure a notice reading as follows: “This Structure Is Unsafe and Its Occupancy Has Been Prohibited by the Code Official.  It shall be unlawful for any person to enter such a structure except for the purpose of securing the structure, making the required repairs, removing the hazardous conditions or demolishing the same.

  - Temporary safeguards.  Notwithstanding other provisions of the code, whenever, in the opinion of the code official, there is imminent danger due to an unsafe condition, the code official shall order the necessary work to be done, including the boarding up of openings, to render such structure temporarily safe whether or not the legal procedure herein described has been instituted; and shall cause such other action to be taken as the code official deems necessary to meet such an emergency.

  - Closing streets.  When necessary for public safety, the code official shall temporarily close structures and close, or order the authority having jurisdiction to close, sidewalks, streets, public ways and places adjacent to unsafe structures, and prohibit the same from being utilized.

  - Emergency repairs.  For the purposes of this of this section, the code official shall employ the necessary labor and materials to perform the required work as expeditiously as possible.

  - Costs of emergency repairs.  Costs incurred in the performance of emergency work shall be paid by the jurisdiction.  The legal counsel for Goshen Township shall institute appropriate action against the owner of the premises where the unsafe structure is or was located for the recovery of such costs.

SECTION 110

DEMOLITION

  - General.  The code official shall order the owner of any premises upon which is located any structure, which in the code official’s judgment is so old, dilapidated or has become so out of repair as to be dangerous, unsafe, unsanitary or otherwise unfit for human habitation or occupancy, and such that it is unreasonable to repair the structure, to demolish and remove such structure; or if such structure is capable of being made safe and sanitary or to demolish and remove at the owner’s option.  Or where there has been a cessation of normal construction of any structure for a period of more than two years, to demolish and remove such structure.

  - Notices.  All notices shall comply with Section 107.

  - Failure to comply.  If the owner of a premises fails to comply with a demolition order within the time prescribed, the code official shall cause the structure to be demolished and removed, either through an available public agency or by contract or arrangement with private persons, and the cost of such demolition and removal shall be charged against the real estate.

  - Salvage materials.  When any structure has been ordered demolished and removed, the Goshen Township Board of Trustees or other designated officer under said contract or arrangement aforesaid shall have the right to sell the salvage and valuable materials at the price obtainable.  The net proceeds of such sale, after deducting the expenses of such demolition and removal, shall be promptly remitted with a report of such sale or transaction, including the items of expense and the amounts deducted , for the person who is entitled thereto, subject to any order of a court.  If such a surplus does not remain to be turned over, the report shall so state.

# SECTION 111
MEANS OF APPEAL

111.1 Application for appeal. Any person affected by a decision of the code official or a notice or order issued under this code shall have the right to appeal to the board of appeals, provided that a written application for appeal is filed within 20 days after the day the decision, notice or order was served. An application for appeal shall be based on a claim that the true intent of this code or the rules legally adopted thereunder have been incorrectly interpreted, the provisions of this code do not fully apply, or the requirements of this code are adequately satisfied by other means.

111.2 Membership of the board. The board of appeals shall consist of the five township residents who serve as the Township Board of Zoning Appeals.

111.2.1 Alternate members. (RESERVIED)

111.2.2 Chairman.  The board shall annually select one of its members to serve as chairman.

111.2.3 Disqualification of member.  A member shall not hear an appeal in which that member has any personal, professional or financial interest.

111.2.4 Secretary.  The chief administrative officer shall designate a qualified clerk to serve as secretary to the board. The secretary shall file a detailed record of all proceedings in the office of the chief administrative officer.

111.2.5 Compensation of members.  (RESERVED)

111.3 Notice of meeting.  The board shall meet upon notice from the chairman or at stated periodic meetings.

111.4 Open hearing. All hearings before the board shall be open to the public. The appellant, the appellant's representative, the code official, and any person whose interests are affected shall be given an opportunity to be heard.

111.4.1 Procedure. The board shall adopt and make available to the public through the secretary, procedures under which a hearing will be conducted. The procedures shall not require compliance with strict rules of evidence but shall mandate that only relevant information be received.

111.5    Postponed hearing. When five members are not present to hear an appeal, either the appellant or the appellant's representative shall have the right to request a postponement of the hearing.

111.6  Board decision. The board shall modify or reverse the decision of the code official by a concurring vote of three members.

111.6.1 Resolution. The decision of the board shall be by motion. Certified copies shall be furnished to   
 the appellant and to the code official.

111.6.2 Administration. The code official shall take immediate action in accordance with the decision of 
 the board.

# 111.7    Court review. Any person, who participates in the hearing before the Board of Appeals or who can demonstrate that they will be especially damaged by the decision of the Board of Appeals shall have the right to apply to the appropriate court in accordance with the provisions of Chapter 2506 of the Ohio Revised Code. Application for review shall be made in the manner and time required by law following the making of the decision.

111.8	Stays of enforcement.  Appeals of notice and orders (other than Imminent danger notices) shall stay the enforcement of the notice and order until the appeal is heard by the appeals board.

# CHAPTER 2

DEFINITIIONS

SECTION 2O1

GENERAL

201.1 	Scope. Unless otherwise expressly stated, the following terms shall, for the purposes of this code, have the meanings shown in this chapter.

201.2 	Interchangeability. Words stated in the present tense include the future; words stated in the masculine gender include the feminine and neuter; the singular number includes the plural and the plural, the singular.

201.3	Reserved.

201.4 	Terms not defined. Where terms are not defined through the methods authorized in this section, such terms shall have ordinarily accepted meanings such as the context implies.

201.5 	Parts. Whenever the words “dwelling unit,” “dwelling,” “premises,” “building,” “rooming house,” “rooming unit,” “housekeeping unit”, or “story” are stated in this code, they shall be construed as though they were followed by the words “or any part thereof.”

SECTION 202

GENERAL DEFINITIIONS

APPROVED.  Approved by the code official.

BASEMENT. That portion of a building which is partly or completely below grade.

CODE OFFICIAL. The official in charge of the administration and enforcement of this code, or any duly authorized representative.

CONDEMN.  To adjudge unfit for occupancy.

DWELLING UNIT. A single unit providing complete independent living facilities for one or more persons, including permanent provisions for living, sleeping, eating, cooking and sanitation.

EASEMENT. That portion of land or property reserved for present or future use by a person or agency other than legal fee owner(s) of the property. The easement shall be permitted to be for use under, on or above a said lot or lots.

EXTERIOR PROPERTY. The open space on premises and adjoining property under the control of owners or operators of such premises.

EXTERMINATION. The control and elimination of insects, rats or other pests by eliminating their harborage places; by removing or making inaccessible materials that serve as their food; by poison spraying, fumigating, trapping or by any other approved pest elimination methods.

GARBAGE. The animal or vegetable waste resulting from the handling, preparation, cooking and consumption of food.

GUARD.  A building component or a system of building components located at or near the open sides of elevated walking surfaces that minimizes the possibility of a fall from the walking surface to a lower level.

HABITABLE SPACE.  Space in a structure for living, sleeping, eating or cooking. Bathrooms, toilet rooms, halls, storage or utility space, similar areas and not considered habitable spaces.

HOUSEKEEPING UNIT.  A room or group of rooms forming a single habitable space equipped and intended to be used for living, sleeping, cooking and eating which does not contain, within such a unit, a toilet, lavatory and bathtub or shower.

IMMINENT DANGER. A condition which could cause serious or life-threatening injury or death at any time.

INFESTATION.  The presence, within or contiguous to, a structure or premises of insects, rats, vermin or other pests.

INOPERABLE MOTOR VEHICLE.  A vehicle which cannot be driven upon the public streets for reasons including, but not limited to, it being unlicensed, wrecked, abandoned, in a state of disrepair, or incapable of being moved under its own power.

LET FOR OCCUPANCY OF LET.  To permit, provide of offer possession or occupancy of a dwelling, dwelling unit, building, premises or structure by a person who is or is not the legal owner of record thereof, pursuant to a written or unwritten lease, agreement or license, or pursuant to a recorded or unrecorded agreement of contract fro the sale of land.

OCCUPANCY.  The purpose for which a building or portion thereof is utilized or occupied.

OCCUPANT.  Any individual living or sleeping in a building, or having possession of a space within a building.

OPENABLE AREA.  That part of a window, skylight or which is available for unobstructed ventilation and which opens directly to the outdoors.

OPERATOR.  Any person who has charge, care or control of a structure or premises which is let or offered for occupancy.

OWNER. Any person, agent, operator, firm or corporation having a legal or equitable interest in the property; or recorded in the official records of the state, county or municipality as holding title to the property; or otherwise having control of the property, including the guardian of the estate of any such person, and the executor or administrator of the estate of such person if ordered to take possession of real property by a court.

PERSON.  An individual, corporation, partnership or any group acting as a unit.

PREMISES.  A lot, plot or parcel of land, easement or public way, including any structures thereon.

PUBLIC WAY.  Any street, alley or similar parcel of land essentially unobstructed from the ground to the sky, which is deeded, dedicated or otherwise permanently appropriated by the public for public use.

ROOMING HOUSE. A building arranged or occupied for lodging, with or without meals, for compensation and not occupied as a one- or two-family dwelling.

ROOMING UNIT.  Any room or group of rooms forming a single habitable unit occupied or intended to be occupied for sleeping or living, but not for cooking purposes.

RUBBISH. Combustible and noncombustible waste materials except garbage; the term shall include the residue from burning of wood, coal, coke and other combustible materials, tires, paper, rags, cartons, boxes, wood, excelsior, rubber, leather, tree branches, yard trimmings, tin cans, metals, mineral matter, glass, crockery and dust and other similar materials. The term shall also include inoperable and broken appliances, electronic and household equipment.

SLEEPING UNIT.  A room or space in which people sleep, which can also include permanent provisions for living, eating and either sanitation or kitchen facilities, but not both.  Such rooms and spaces that are also part of a dwelling unit are not sleeping units.

STRICT LIABILITY OFFENSE.  An offense in which the prosecution in a legal proceeding is not required to prove criminal intent as a part of its case.  It is enough to prove that the defendant either did an act which was prohibited, or failed to do an act which the defendant was legally required to do.

STRUCTURE.  Anything constructed or erected that requires location on the ground, or attachment to something having location on the ground.

TENANT.  A person, corporation, partnership or group: whether or not the legal owner of record, occupying a building or portion thereof as a unit.

TOILET ROOM.  A room containing a water closet or urinal but not a bathtub or shower.

VENTILATION.  The natural or mechanical process of plying conditioned or unconditioned air to, or removing air from, any space.

WORKMANLIKE.  Executed in a skilled manner; e.g., generally plumb, level, square, in line, undamaged and without marring adjacent work.

YARD.   An open space on the same lot with a structure.

CHAPTER 3

GENERAL REQUIREMENTS

SECTION 301

GENERAL

301.1 	Scope. The provisions of this chapter shall govern the minimum conditions and the responsibilities of persons for maintenance of residential structures, equipment and exterior property.

301.2 	Responsibility. The owner of the premises shall maintain the structure(s) and exterior property in compliance with these requirements, except as otherwise provided for in this code. A person shall not occupy as owner-occupant or permit another person to occupy premises which are not in a sanitary and safe condition and which do not comply with the requirements of this chapter. Occupants of a dwelling unit are responsible for keeping in a clean, sanitary and safe condition that part of the premises which they occupy and control.

301.3 	Vacant structures and land. All vacant structures and premises thereof or vacant land shall be maintained in a clean, safe, secure and sanitary condition as provided herein so as not to cause a blighting problem or adversely affect the public health, safety, or welfare.

SECTION 302

EXTERIOR PROPERTY AREAS

302.1	Sanitation.  All exterior property and premises shall be maintained in a clean, safe and sanitary condition. The occupant shall keep that part of the exterior property which such occupant occupies or controls in a clean and sanitary condition.

302.2	Grading and drainage.   All premises shall be graded and maintained to prevent the erosion of soil and to prevent the accumulation of stagnant water thereon, or within any structure located thereon.

Exception:  Approved retention areas and reservoirs.

302.3 	Driveways and walkways.  All driveways, walkways, stairs and similar areas shall be kept in a proper state of repair, and maintained free from hazardous conditions.

  - Weeds.  All premises and exterior property not used for agriculture shall be maintained free from weeds and plant growth in excess of eight (8) inches in height. All noxious weeds shall be prohibited.  Weeds shall be defined as all grasses, annual plants and vegetation, other than trees or shrubs provided, however, this term shall not include cultivated flowers and gardens.  Upon failure of the owner of agent having charge of a property to cut and destroy weeds after service of a notice of violation, they shall be subject to prosecution in accordance with Section 106.3 and as prescribed by the authority having jurisdiction.  Upon failure to comply with the notice of violation, any duly authorized employee of Goshen Township or contractor hired by Goshen Township shall be authorized to enter upon the property in violation and cut and destroy the weeds growing thereon, and the costs of such removal shall be paid by the owner or agent responsible for the property.

  - Rodent harborage.  All structures and exterior property shall be kept free from rodent harborage and infestation. Where rodents are found, they shall be promptly exterminated by approved processes which will not be injurious to human health. After extermination, proper precautions shall be taken to eliminate rodent harborage and prevent reinfestation.

  - Exhaust fans.  Pipes, ducts, conductors, fans or blowers shall not discharge gases, steam, vapor, hot air, grease, smoke, odors or other gaseous or particulate wastes directly upon abutting or adjacent public or private property or that of another tenant.

  - Accessory structures.  All accessory structures, including detached garages, fences and walls, shall be maintained structurally sound and in good repair.

302.8 	Motor vehicles. Except as provided for in other regulations, no inoperative or unlicensed motor vehicle shall be parked, kept or stored on any premises, and no vehicle shall at any time be in a state of major disassembly, disrepair, or in the process of being stripped or dismantled.  Painting of vehicles is prohibited unless conducted inside an approved spray booth.

Exception: A vehicle of any type is permitted to undergo major overhaul, including body work, provided that such work is performed inside a structure or similarly enclosed area designed and approved for such purposes.

  - Defacement of property.  No person shall willfully or wantonly damage, mutilate or deface any exterior surface of any structure or building on any private or public property by placing thereon any marking, carving or graffiti.  It shall be the responsibility of the owner to restore said surface to an approved state of maintenance and repair.

SECTION 303

SWIMMING POOLS, SPAS AND HOT TUBS

303.1 	Swimming Pools. Swimming pools shall be maintained in a clean, sanitary condition, and in good repair.

  - Enclosures. Private swimming pools, hot tubs and spas containing water more than 24 inches (610mm) in depth shall be completely surrounded by a fence or barrier at least 48 inches (1219mm) in height above the finished ground level measured on the side of the barrier away from the pool. Gates and doors in such barriers shall be self-closing and self-latching. Where the self-latching device is less then 54 inches (1372mm) above the bottom of the gate, the release mechanism shall be located on the pool side of the gate. Self-closing and self-latching gates shall be maintained such that the gate will positively close and latch when released from an open position of 6 inches (152mm) from the gatepost. No existing pool enclosure shall be removed, replaced or changed in a manner that reduces its effectiveness as a safety barrier.

Exception:  Spas or hot tubs with a safety cover that complies with ASTM F 1346 shall be 	exempt from the provisions of this section.

SECTION 304

EXTERIOR STRUCTURE

304.1 	General.  The exterior of a structure shall be maintained in good repair, structurally sound and sanitary so as not to pose a threat to the public health, safety or welfare.

304.2	Protective treatment.  All exterior surfaces, including but not limited to, doors, door and window frames, cornices, porches, trim, balconies, decks and fences shall be maintained in good condition.  Exterior wood surfaces, other than decay-resistant woods, shall be protected from the elements and decay by painting or other protective covering or treatment.  Peeling, flaking and chipped paint shall be eliminated and surfaces repainted.  All siding and masonry joints as well as those between the building envelope and the perimeter of windows, doors, and skylights shall be maintained weather resistant and water tight.  All metal surfaces subject to rust and corrosion shall be coated to inhibit such rust and corrosion and all surfaces with rust and corrosion shall be stabilized and coated to inhibit future rust and corrosion.  Oxidation stains shall be removed from exterior surfaces.  Surfaces designed for stabilization by oxidation are exempt from this requirement.

304.3 	Premises identification.  Buildings shall have approved address numbers placed in a position to be plainly legible and visible from the street or road fronting the property.  These numbers shall contrast with their background.  Address numbers shall be Arabic numerals or alphabet letters.  Numbers shall be a minimum of 4 inches (102 mm) high with a minimum stroke width of 0.5 inch (12.7 mm).

304.4	Structural members. All structural members shall be maintained free from deterioration, and shall be capable of safely supporting the imposed dead and live loads.

304.5 	Foundation walls. All foundation walls shall be maintained free from open cracks and breaks and kept in such a condition so as to prevent the entry of rodents and other pests.

304.6	Exterior walls. All exterior walls shall be free from holes, breaks, and loose or rotting materials; and maintained weatherproof and properly surface coated where required to prevent deterioration.

304.7	Roofs and drainage.  The roof and flashing shall be sound, tight and not have defects that admit rain.  Roof drainage shall be adequate to prevent dampness or deterioration in the walls or interior portion of the structure.  Rood drains, gutters and downspouts shall be maintained in good repair and free from obstructions.  Roof water shall not be discharged in a manner that creates a public nuisance.

304.8 	Decorative features. All cornices, belt courses, corbels, terra cotta trim, wall facings and similar decorative features shall be maintained in good repair with proper anchorage and in a safe condition.

304.9	Overhang extensions.  All overhung extensions including, but not limited to canopies, marquees, signs, metal awnings, fire escapes, standpipes and exhaust ducts shall be maintained in good repair and be properly anchored so as to be kept in a sound condition.  When required, all exposed surfaces of metal and wood shall be protected from the elements and against decay or rust by periodic application of weather-coating materials, such as paint or similar surface treatment.

304.10	Stairways, decks, porches and balconies.  Every exterior stairway, deck, porch and balcony, and all appurtenances attached thereto, shall be maintained structurally sound, in good repair, with proper anchorage and capable of supporting the imposed loads.

304.11	Chimneys and towers.  All chimneys, cooling towers, smoke stacks, and similar appurtenances shall be maintained structurally safe and sound, and in good repair.  All exposed surfaces of metal or wood shall be protected from the elements and against decay and rust by periodic application of weather-coating materials, such as paint or similar surface treatment.

304.12 	Handrails and guards. Every handrail and guard shall be firmly fastened and capable of supporting normally imposed loads and shall be maintained in good condition.

304.13 	Window, skylight and door frames.  Every window, skylight, door and frame shall be kept in sound condition, good repair and weather tight.

304.13.1 Glazing.  All glazing materials shall be maintained free form cracks and holes.

304.13.2 Openable windows. Every window, other than a fixed window, shall be easily openable and capable of being held in position by window hardware.

304.14	Insect screens.  Any screen used at any time in a door, window or other outside opening for ventilation shall be maintained with tightly fitting screens with no rips or tears therein.

  - Doors. All exterior doors, door assemblies and hardware shall be maintained in good condition.  Locks at all entrances to dwelling units and sleeping units shall tightly secure the door.

  - Basement hatchways.  Every basement hatchway shall be maintained to prevent the entrance of rodents, rain and surface drainage water.

  - Guards for basement windows. Every basement window that is openable shall be supplied with rodent shields, storm windows or other approved protection against the entry of rodents.

  - Building security. Doors, windows or hatchways for dwelling units, room units or housekeeping units shall be provided with devices designed to provide security for the occupants and property within.

    - Doors. Doors providing access to a dwelling unit, rooming unit or housekeeping unit that is rented, leased or let shall be equipped with a deadbolt lock designed to be readily openable from the side from which egress is to be made without the need for keys, special knowledge or effort and shall have a lock throw o f not less than 1 inch (25 mm).  Such deadbolt locks shall be installed according to the manufacturer’s specifications and maintained in good working order.  For the purpose of this section, a sliding bolt shall not be considered an acceptable deadbolt lock.

304.18.2 Windows.  Openable windows located in whole or in part with 6 feet (1828 mm) above ground level or a walking surface below that provide access to a dwelling unit, rooming unit or housekeeping unit that is rented, leased or let shall be equipped with a window sash locking device.

    - Basement hatchways. Basement hatchways that provide access to a dwelling unit, rooming unit or housekeeping unit tat is rented, leased or let shall be equipped with devices that secure the units from unauthorized entry.

SECTION 305

INTERIOR STRUCTURE

Reserved

SECTION 306

HANDRAILS AND GUARDRAILS

  - General. Every exterior and interior flight of stairs having more than four risers shall have a handrail on one side of the stair, landing, balcony, porch, deck, ramp or other walking surface which is more than 30 inches (762 mm) above the floor or grade shall have guards.  Handrails shall not be less than 30 inches (762 mm) high or more than 42 inches (1067 mm) high measured vertically above the nosing of the tread or above the finished floor of the landing or walking surfaces.  Guards shall not be less than 30 inches (762 mm) high above the floor of the landing, balcony, porch, deck, or ramp or other walking surface.

Exception:  Guards shall not be required where exempted by the Clermont County Building Code.

SECTION 307

RUBBUISH AND GARBAGE

307.1 	Accumulation of rubbish or garbage. All exterior property and premises, and the interior of every structure, shall be free from any accumulation of rubbish or garbage.

  - Disposal of rubbish. Every occupant of a structure shall dispose of rubbish in a clean and sanitary manner by placing such rubbish in approved containers.

    - Rubbish storage facilities. The owner of every occupied premises shall supply approved containers for rubbish, and the owner of the premises shall be responsible for the removal of rubbish.

307.2.2 Refrigerators. Refrigerators and similar equipment not in operation shall not be discarded, abandoned or stored on premises without first removing the doors.

307.3 	Disposal of garbage. Every occupant of a structure shall dispose of garbage in a clean and sanitary manner by placing such garbage in an approved garbage disposal facility or approved garbage containers.

307.3.1 Garbage facilities.  The owner of every dwelling unit shall supply one of the following: an approved mechanical food waste grinder in each dwelling unit; an approved incinerator unit in the structure available to the occupants in each dwelling unit; or an approved leakproof, covered, outside garbage container.

307.3.2 	Containers. The operator of every establishment producing garbage shall provide, and at all times cause to be utilized, approved leak proof containers provided with close-fitting covers for the storage of such materials until removed from the premises for disposal.

SECTION 308

EXTERMINATION

  - Infestation.  All structures shall be kept free from insect and rodent infestation.  All structures in which insects or rodents are found shall be promptly exterminated by approved processes that will not be injurious to human health.  After extermination, proper precautions shall be taken to prevent reinfestation.

  - Owner. The owner of any structure shall be responsible for extermination within the structure prior to renting or leasing the structure.

  - Single occupant. The occupant of a one-family dwelling or of a single-tenant nonresidential structure shall be responsible for extermination on the premises.

  - Multiple occupancy.  The owner of a structure containing two or more dwelling units, a multiple occupancy, a rooming house or a nonresidential structure shall be responsible for extermination in the public or shared areas of the structure and exterior property.  If infestation is caused by failure of an occupant to prevent such infestation in the area occupied, the occupant shall be responsible for extermination.

  - Occupant.  The occupant of any structure shall be responsible for the continued rodent and pest-free condition of the structure.

Exception: Where the infestations are caused by defects in the structure, the owner shall be responsible for extermination.

SECTION 309

SIGNAGE

  - Prohibited and Obsolete Signs. All prohibited and obsolete signs as defined by the Goshen Township Zoning Resolution and signs with expired zoning permits shall be removed or replaced pursuant to the applicable sections of the Zoning Resolution or enforced consistent with the procedures of this code.

  - Sign Maintenance. All signage shall be maintained in good condition and shall not show evidence of deterioration.

  - Visibility Triangles. All signage shall be located outside of all visibility triangles at intersections and ingress/egress points, and shall not be located in such a manner as to constitute a traffic or safety hazard.